#### 100BASE-T1 / SPE Pass-Through Testing on Various Connectivity Schemes

In order to validate the concept of Single Pair Ethernet (SPE) connectivity, a series of measurements were performed under real hardware conditions.

Used devices:

- Raspberry Pi 3B+ RPi OS

- PC/Laptop with Windows 11

- EV02N47A Evaluation Board, LAN8770M, Interface, 100BASE-TX and 100BASE-T1 Ethernet PHY

- iPerf3 testing software tool - used for TCP throughput measurement

The following tests were performed with positive results:

- Windows ↔ 100BASE-TX ↔ SPE PHY ↔ 100BASE-T1 (2-wire) ↔ SPE PHY ↔ 100BASE-TX ↔ Raspberry Pi over single twisted pair (STP)

- Windows ↔ 100BASE-TX ↔ SPE PHY ↔ 100BASE-T1 (2-wire) ↔ SPE PHY ↔ 100BASE-TX ↔ Raspberry Pi over single NON twisted pair (NSTP)

- Windows ↔ 100BASE-TX ↔ SPE PHY ↔ 100BASE-T1 (2-wire) ↔ SPE PHY ↔ 100BASE-TX ↔ Raspberry Pi passing 3 cases via DIN BUS (same we will use)

## Test (1) Windows ↔ 100BASE-TX ↔ SPE PHY ↔ 100BASE-T1 (2-wire) ↔ SPE PHY ↔ 100BASE-TX ↔ Raspberry Pi over single twisted pair (STP)

Purpose: This test validates the nominal 100BASE-T1 / SPE link performance using a single twisted pair. This is the preferred physical cabling method because twisting helps reduce common-mode noise pickup and improves electromagnetic immunity.

Physical configuration: The Windows 11 PC was connected to the 100BASE-TX side of the first EV02N47A board. The two-wire 100BASE-T1 link was routed between the two EV02N47A boards using a single twisted pair. The second board converted the link back to 100BASE-TX and connected to the Raspberry Pi 3B+.

Measurement procedure: The Raspberry Pi 3B+ acted as the iperf3 server, and the Windows 11 PC acted as the iperf3 client. The Windows command was bound to the correct Ethernet interface to ensure that traffic passed through the SPE link under test.

Acceptance criterion: A stable TCP throughput close to 94-95 Mbit/s is expected for a correctly operating 100 Mbit/s Ethernet link, due to Ethernet/IP/TCP overhead.

Test command used on Windows: .\iperf3.exe -c 192.168.1.247 -B 192.168.1.163

Measured results:
• Average TCP throughput: 95.1 Mbit/s
• Data transfer: ~113–114 MBytes / 10 seconds
• Packet loss: none observed
• Link behavior: stable

Assessment: PASS
The result matches expected 100BASE-T1 TCP performance (~95 Mbit/s).

Transfer: ~113–114 MBytes / 10 seconds

Packet loss: none visible in iperf result

Connection: stable

Result: PASS

This result matches the expected performance of a 100BASE-T1 link. Measured TCP throughput (~95 Mbit/s) is consistent with protocol overhead and confirms optimal link utilization.

Test confirms:

100BASE-T1 / SPE physical link: OK

100BASE-TX to 100BASE-T1 conversion: OK

Throughput: OK

Stability: OK

## Test (2) Windows ↔ 100BASE-TX ↔ SPE PHY ↔ 100BASE-T1 (2-wire) ↔ SPE PHY ↔ 100BASE-TX ↔ Raspberry Pi over single NON twisted pair (NSTP)

Purpose: This test checks link robustness when the 100BASE-T1 connection is made using two non-twisted wires. This is a non-ideal wiring condition and is useful as a practical stress test for the PHY and the physical interconnection.

Physical configuration: The overall topology remained the same as Test (1), but the SPE segment between the two EV02N47A boards was changed from a twisted pair to two non-twisted wires.

Measurement procedure: The same iperf3 command, endpoints, and IP addressing were used. This allows a direct comparison between twisted-pair and non-twisted wiring under the same measurement conditions.

Engineering interpretation: Achieving the same throughput as the twisted-pair test indicates that, over the tested distance and bench conditions, the PHY link margin was sufficient even with non-ideal cabling. For the final industrial design, a twisted pair is still recommended.

Test command used on Windows: .\iperf3.exe -c 192.168.1.247 -B 192.168.1.163

Measured results:
• Average TCP throughput: 95.1 Mbit/s
• Data transfer: ~113–114 MBytes / 10 seconds
• Packet loss: none observed
• Link behavior: stable

Assessment: PASS
The result matches expected 100BASE-T1 TCP performance (~95 Mbit/s).

Transfer: ~113–114 MBytes / 10 seconds

Packet loss: none visible in iperf result

Connection: stable

Result: PASS

This result matches the expected performance of a 100BASE-T1 link. Measured TCP throughput (~95 Mbit/s) is consistent with protocol overhead and confirms optimal link utilization.

Test confirms:

100BASE-T1 / SPE physical link: OK

100BASE-TX to 100BASE-T1 conversion: OK

Throughput: OK

Stability: OK

## Test (3)  Windows ↔ 100BASE-TX ↔ SPE PHY ↔ 100BASE-T1 (2-wire) ↔ SPE PHY ↔ 100BASE-TX ↔ Raspberry Pi passing 3 cases via DIN BUS (same we will use)

Purpose: This test validates the intended installation scenario, where the SPE connection passes through the DIN bus structure and multiple connector/contact points.

Physical configuration: The 100BASE-T1 two-wire path was routed through three DIN bus cases/segments, representing the same style of modular interconnection expected in the final product architecture.

Measurement procedure: The same iperf3 setup was used again so that the DIN bus result could be compared directly against the direct twisted-pair and non-twisted-pair measurements.

Engineering interpretation: Maintaining approximately 95 Mbit/s confirms that the DIN bus routing and additional connector transitions did not introduce measurable throughput degradation in this test setup.

Test command used on Windows: .\iperf3.exe -c 192.168.1.247 -B 192.168.1.163

Measured results:
• Average TCP throughput: 95.1 Mbit/s
• Data transfer: ~113–114 MBytes / 10 seconds
• Packet loss: none observed
• Link behavior: stable

Assessment: PASS
The result matches expected 100BASE-T1 TCP performance (~95 Mbit/s).

Transfer: ~113–114 MBytes / 10 seconds

Packet loss: none visible in iperf result

Connection: stable

Result: PASS

This result matches the expected performance of a 100BASE-T1 link. Measured TCP throughput (~95 Mbit/s) is consistent with protocol overhead and confirms optimal link utilization.

Test confirms:

100BASE-T1 / SPE physical link: OK

100BASE-TX to 100BASE-T1 conversion: OK

Throughput: OK

Stability: OK

Overall assessment: All three tested configurations reached approximately 95.1 Mbit/s, which is the expected practical TCP throughput limit for a 100 Mbit/s Ethernet link. The results confirm that the 100BASE-TX to 100BASE-T1 conversion, the two-wire SPE segment, and the DIN bus routing are suitable for the tested application scenario.

Recommendation: For the final product implementation, it is recommended that the SPE (Single Pair Ethernet) segment be designed using a controlled and repeatable wiring approach. The preferred configuration is a single twisted pair with well-defined polarity and consistent connector assignment across all modules. This ensures optimal signal integrity, improved electromagnetic compatibility (EMC), and predictable system behavior under varying operating conditions.

The measurements conducted confirm that the proposed DIN bus routing concept is functionally valid and does not introduce measurable degradation in throughput under the tested conditions.

Based on the performed tests, it is confirmed that SPE communication can be reliably implemented within the selected DIN bus housing architecture. This includes scenarios where short interconnections between modules are realized using non-twisted conductors. The key factor enabling this behavior is the use of a daisy-chain switch topology within each housing module. As a result, the non-twisted segments are physically very short and do not significantly affect signal integrity or link performance.

The primary objective of these tests was to evaluate whether short non-twisted SPE connections could negatively impact throughput or link stability. The results demonstrate that, within the tested configuration, no measurable degradation in TCP throughput (~95 Mbit/s) or link stability was observed. This indicates sufficient link margin and robustness of the PHY layer under these specific conditions.

In conclusion, the combination of SPE PHYs, short interconnect segments, and a daisy-chain switch architecture is suitable for the intended DIN bus modular system. The solution meets the expected performance targets and provides sufficient robustness for the defined application, subject to successful completion of additional validation testing prior to final product release.

Figure 1 – Test workbench and topology
Complete setup with PC, PHY boards and Raspberry Pi. Link LEDs confirm correct operation.

Figure 2 – Twisted pair (STP) SPE connection
Proper twisted wiring ensures optimal signal integrity and EMC performance.

Figure 3 – Non-twisted pair (NSTP) connection
Non-ideal wiring still maintains stable link, showing PHY robustness.

Figure 2 – Twisted pair (STP) SPE connection
Proper twisted wiring ensures optimal signal integrity and EMC performance.

Figure 4 – DIN bus routing
Multi-segment connection through DIN modules shows no performance degradation.